NG Actuator v14

10 Nm Calibration Benchmark — Architecture, DMA Migration, RTOS & Memory Guide

Baseline: v13.2 CalibrationRestoreFix | STM32C562RE | TMC6460 | FreeRTOS | IAR EWARM

# 1. Benchmark status and scope

This version is derived from the working v13.2 calibration project and is designated as the calibration benchmark for the 10 Nm actuator. The legacy source enum ACTUATOR_TYPE_10KG is retained to avoid breaking interfaces, but command A is documented as the 10 Nm benchmark profile.

Preserve the proven SPI2 + external CSN transport behavior.

Preserve TMC6460 run motion mode 0x00002786 and initialization mode 0x00002386.

Preserve the 400 Hz normal position sampling architecture and sole-owner MotorService rule.

Preserve G calibration and X/Y soft-end state machines unless a change is explicitly validated.

# 2. Requested configuration changes

Profile A speed fields are both changed to 9,000,000 raw:

{ 1000, 1200, 1200, 9000000, 9000000 }

The last two fields are default_velocity_raw and maximum_velocity_raw. B/C are intentionally unchanged because they are not part of this 10 Nm benchmark.

Calibration maximum leg duration is increased to 150 seconds:

#define ACTUATOR_CALIBRATION_MAX_LEG_TIME_MS 150000U

Soft-stall creep begins at 98.00% of calibrated stroke:

#define ACTUATOR_SOFT_STALL_CREEP_START_PERCENT_X100 9800U

Periodic debug output is reduced to one compact status line every 1 second. Detailed memory output is on command M; soft parameters remain on P; explicit controller diagnostic refresh remains on R.

# 3. System architecture

Application
  app.c / actuator_profile.c / calibration / reference / soft_stall / runtime_memory
            |
            v
TMC6460 motor middleware -> register API -> transport selector
            |                         |
            |                    UART or SPI
            |                         |
            +-------------------------+
                         |
                 RTOS-aware I/O drivers
                 UART DMA / SPI DMA
                         |
                         v
               BSP: GPIO/RCC/NVIC/LPDMA1
                         |
                         v
                    STM32C562RE

The application layer contains no DMA channel numbers, GPIO alternate functions or IRQ names. Those remain in hardware_config.h and BSP. This is the main scalability rule: actuator behavior should not change when the transport implementation changes.

# 4. DMA design and channel allocation

#define HW_UART_DMA_ENABLE 1U
#define HW_SPI_DMA_ENABLE  1U

| LPDMA1 channel | Purpose | Peripheral request |
| --- | --- | --- |
| CH0 | SPI2 RX | SPI2_RX |
| CH1 | SPI2 TX | SPI2_TX |
| CH2 | USART2 debug RX | USART2_RX |
| CH3 | USART2 debug TX | USART2_TX |
| CH4 | Optional USART1 TMC RX | USART1_RX |
| CH5 | Optional USART1 TMC TX | USART1_TX |

All DMA IRQs use FreeRTOS-compatible preemption priority 5. This is important because completion callbacks give FreeRTOS semaphores or write to a stream buffer from ISR context.

## 4.1 SPI DMA sequence

MotorService -> SPI_IF_Transfer
  take SPI mutex
  clear done semaphore
  CSN LOW
  HAL_SPI_TransmitReceive_DMA()
  task blocks on done semaphore
       |
       +-- LPDMA1 IRQ -> HAL DMA -> HAL SPI completion callback
                                  -> CSN HIGH
                                  -> xSemaphoreGiveFromISR(done)
  task resumes
  release SPI mutex

The external CS behavior is unchanged from the proven SPI benchmark. Each TMC6460 48-bit datagram gets one CS pulse. Pipelined reads/writes still use two separate SPI_IF_Transfer calls, therefore two separate CS pulses.

## 4.2 UART DMA sequence

UART transmit uses HAL_UART_Transmit_DMA. Transactional receive uses HAL_UART_Receive_DMA. Debug command reception uses HAL_UART_ReceiveToIdle_DMA into a 32-byte chunk. On an IDLE or full-buffer event, the ISR copies the valid bytes into the existing FreeRTOS stream buffer and rearms DMA.

USART2 RX DMA chunk -> HAL UART callback -> xStreamBufferSendFromISR -> DebugCommandTask

# 5. DMA migration plan to avoid functional regression

1. Keep the v13.2 .zip, .hex/.out and .map as an immutable rollback baseline.

2. First compile v14 with HW_UART_DMA_ENABLE=0 and HW_SPI_DMA_ENABLE=0. Validate only the requested profile/timeout/creep/debug changes.

3. Enable UART DMA only. Verify A/V/W/Z/G/X/Y commands are received, long calibration prints do not freeze, and M reports rxDrop=0.

4. Enable SPI DMA. Before motor motion, verify CHIP_ID, a non-zero-address register read, manual shaft position/velocity feedback, and external-CS waveform.

5. Run A, V, Z, W, Z at 9,000,000 raw on the validated current-limited bench.

6. Run a complete G calibration. Compare endpoint spread and stroke against v13.2.

7. Run repeated X/Y soft-end operations. Confirm creep begins at 98% and contact remains safe.

8. Stress run and inspect M: DMA errors/timeouts should remain zero or isolated; UART dropped bytes must be zero; task stack margins must remain healthy.

The supplied v14 project has both DMA switches enabled. The staged switch-off/on sequence above is the recommended bring-up method if any regression appears.

# 6. FreeRTOS task architecture

| Task | Priority | Stack words | Approx bytes | Responsibility |
| --- | --- | --- | --- | --- |
| MotorService | 4 | 2048 | 8192 | Only owner of TMC traffic; commands, 400 Hz position, G calibration, X/Y soft end |
| DebugCommand | 3 | 384 | 1536 | Consumes UART stream; parses commands; queues motor commands; sets abort flags |
| Diagnostics | 1 | 512 | 2048 | Compact status every 1 s; reads cached status only |
| Heartbeat | 1 | 256 | 1024 | Toggles PA5 once per second |
| Timer service | 2 | 128 | 512 | FreeRTOS software-timer daemon |
| Idle | 0 | 128 | 512 | FreeRTOS idle task |

## 6.1 MotorService task

MotorService has the highest application priority because actuator control and feedback timing are the primary real-time functions. It drains the motor command queue before the next position sample. Normal position reads are rate limited to 400 Hz to prevent starvation. Calibration and soft-end operations run synchronously in this task, guaranteeing that no other task can interleave TMC register transactions.

## 6.2 DebugCommand task

DebugCommand blocks on the UART stream buffer. Commands A/B/C/V/W/Z/G/X/Y/R are converted into a motor command queue item except P and M, which are local reporting operations. During long G/X/Y operations, Z sets a volatile abort request so the active MotorService state machine can return the actuator to hold without allowing another task to touch the TMC bus.

## 6.3 Diagnostics and Heartbeat

Diagnostics consumes cached state only. It does not read TMC registers, so reducing or disabling diagnostics cannot affect motor control. Heartbeat is isolated from motor logic and exists only as a liveness indication.

# 7. IPC, mutexes, semaphores and synchronization

| Mechanism | Owner/Users | Purpose |
| --- | --- | --- |
| Motor command queue | DebugCommand -> MotorService | Serializes commands and decouples console timing from TMC timing |
| SPI mutex | SPI_IF | Guarantees one transfer/CS owner; future-proofs shared SPI use |
| SPI done binary semaphore | DMA/SPI ISR -> waiting task | Blocks task without busy polling while DMA moves frame |
| UART mutex | UART_IF per initialized port | Prevents overlapping TX/RX transactions on one USART |
| UART TX/RX binary semaphores | UART DMA callback -> task | Completion synchronization |
| Debug RX stream buffer | UART DMA ISR -> DebugCommand | Buffers command bytes and absorbs bursty console input |
| Abort flags | DebugCommand -> MotorService loop | Asynchronous cancel for long calibration/soft move without TMC multi-owner access |
| Critical sections | App status / CS start | Very short atomic sections only; never around long motion |

No event groups, queue sets or task notifications are currently used by the application even though some are enabled in FreeRTOSConfig. Avoid adding IPC primitives unless there is a clear ownership/timing requirement.

# 8. Memory and stack analysis

Device capacities: 512 KiB Flash and 128 KiB SRAM. FreeRTOS heap_4 reserves 24 KiB (24576 bytes) as a static array. The linker therefore counts the full 24 KiB as RAM even if much of the heap is free at runtime.

| Stack consumer | Words | Approx bytes |
| --- | --- | --- |
| MotorService | 2048 | 8192 |
| Diagnostics | 512 | 2048 |
| DebugCommand | 384 | 1536 |
| Heartbeat | 256 | 1024 |
| Idle | 128 | 512 |
| Timer service | 128 | 512 |
| TOTAL | 3456 | 13824 |

Configured task stack payload is 13,824 bytes. Before TCBs, semaphores, queues, stream-buffer metadata and alignment, about 10,752 bytes of the 24 KiB FreeRTOS heap remain. The exact remaining value is available at runtime with M.

The DMA handles and the 32-byte debug DMA receive chunk are static objects in .bss; they do not consume the FreeRTOS heap. DMA itself does not copy into an extra RTOS heap buffer for TMC SPI transactions.

## 8.1 Runtime command M

MEMORY RTOS: heap total=24576 free=... minEverFree=... usedNow=... peakUsed=...
STACK free words: Motor=... Command=... Diagnostics=... Heartbeat=...
IPC: motorQueue free=.../12
DMA I/O: SPI xfers=... timeout=... err=... | UART tx=... rxBytes=... drop=... err=...

High-water mark is the minimum stack space that has ever remained. Multiply words by 4 for approximate bytes on Cortex-M33. A declining high-water mark is more useful than instantaneous stack pointer inspection.

## 8.2 Exact ROM/Flash and linked RAM after IAR build

Exact post-DMA ROM and RAM cannot be known until IAR links this exact source configuration. v14 includes tools/analyze_iar_map.py. After Rebuild All, run it against the generated .map file. It extracts IAR grand totals and computes percentages against 512 KiB Flash and 128 KiB RAM.

python tools/analyze_iar_map.py <path-to-NG_Actuator.map> memory.json

Interpretation: the .map answers link-time reservation; M answers runtime FreeRTOS consumption. Both are required for a meaningful embedded memory assessment.

# 9. Debug UART policy

The previous very large STAT line is removed. The periodic line is now intentionally small:

STAT p=0 st=1 tmc=0 pos=123456 vel=0 cmd=0 drv=1 Hz=400 cal=1/0 ref=1 soft=0/0 err=0 heap=9736 stkM=1450

R: explicit TMC diagnostic refresh.

P: soft-stall configuration/reference information.

M: detailed RTOS, stack, heap and DMA communication statistics.

G/X/Y still print state transitions and errors because those are needed for bench validation.

# 10. Code walkthrough — recommended reading order

1. hardware_config.h — Single STM32C5 hardware/clock/DMA/NVIC/task configuration source.

2. main.c — Boot order: system clock first, BSP, driver middleware, APP, task creation, scheduler.

3. mx_freertos_app.c — Creates the four application tasks and exposes handles only for runtime diagnostics.

4. app.c — Command parser, command queue routing, MotorService ownership, cached status, diagnostics.

5. actuator_profile.c — Per-actuator torque and velocity configuration; A is the 10 Nm benchmark.

6. actuator_calibration.c — Hard-end G calibration state machine, retries, endpoint averaging, repeatability.

7. actuator_reference.c — Separates reusable stroke/direction from power-cycle-dependent raw position anchor.

8. actuator_soft_stall.c — X/Y cruise -> creep -> contact -> hold state machine.

9. tmc6460_motor.c — Motor-oriented controller operations and working register sequence.

10. tmc6460.c — Generic TMC register read/write and field access.

11. tmc6460_transport.c — TMC UART/SPI frame encoding and SPI pipeline behavior.

12. spi_iface.c — FreeRTOS mutex/semaphore + DMA transfer + external CS lifecycle.

13. uart_iface.c — UART DMA TX/RX, DMA-to-IDLE debug RX and stream-buffer bridge.

14. bsp.c — Actual STM32C5 UART/SPI/LPDMA/GPIO/NVIC initialization.

15. runtime_memory.c — Runtime heap, task high-water and driver counters.

# 11. Acceptance checklist

IAR Rebuild All completes with no linker/ISR/DMA configuration errors.

Debug console accepts commands through DMA RX; M reports dropped RX bytes = 0.

A initializes the 10 Nm benchmark and sets default/max speed to 9,000,000 raw.

V/W/Z work as before and the TMC motion readback remains 0x00002786 while running.

Position/velocity change when shaft moves; normal sample rate remains near 400 Hz.

G completes five iterations; any one leg is allowed up to 150 seconds.

X/Y creep transition occurs at 98.00% and physical contact/hold remains stable.

SPI DMA errors/timeouts do not accumulate under repeated motion.

External CS remains one clean pulse per 48-bit frame.

FreeRTOS stack overflow hook is never reached; M shows useful MotorService and other task margins.

New .map is archived with the benchmark build and analyzed with the included map tool.