# NG Actuator Firmware
Architecture & Auto-Calibration Design

Version 11 - derived from verified v10.4 SPI benchmark

Target: STM32C562RE | IAR EWARM | FreeRTOS | TMC6460

Purpose: automatic 0° / 90° mechanical end calibration using debug command G

Calibration intentionally reaches mechanical end stops at low velocity. First validation should use conservative bench power/current limits.

## 1. Baseline and scope

This version is derived from the verified NG_Actuator_Industry_Architecture_IAR_v10_4_MotorTaskFix benchmark. The benchmark behavior is intentionally preserved:

STM32C562RE, IAR EWARM, FreeRTOS.

TMC6460 host transport selectable by HW_TMC_UART_ENABLE / HW_TMC_SPI_ENABLE.

SPI2 benchmark mapping: PB12 CSN, PB13 SCK, PB14 MISO, PB15 MOSI, external active-low CSN, mode 1, 4.5 MHz.

Debug USART2 on ST-LINK VCP.

TMC6460 run motion value 0x00002786 and idle/configuration value 0x00002386.

V = CW, W = CCW, Z = velocity zero + non-zero hold limit + gate driver kept enabled.

Runtime position sampling limited to 400 Hz so the priority-4 MotorService task does not starve lower-priority tasks.

The v11 addition is automatic mechanical end calibration using debug command G.

## 2. Layered architecture

+------------------------------------------------------------------+
|                         Application Layer                         |
|                                                                  |
|  app.c              actuator_profile.c       actuator_calibration.c
|  command routing    actuator types/limits    calibration algorithm |
+------------------------------+-----------------------------------+
                               |
                               v
+------------------------------------------------------------------+
|                      TMC6460 Middleware                           |
|                                                                  |
|  tmc6460_motor.c    motor configure/run/hold/read motion sample  |
|  tmc6460.c          generic 32-bit register access               |
|  tmc6460_transport  UART/SPI framing + transport selection       |
+------------------------------+-----------------------------------+
                               |
                 +-------------+-------------+
                 |                           |
                 v                           v
          +-------------+              +-------------+
          |  UART_IF    |              |   SPI_IF    |
          | IRQ + RTOS  |              | IRQ + RTOS  |
          +------+------+              +------+------+ 
                 |                            |
                 +-------------+--------------+
                               v
+------------------------------------------------------------------+
|                         BSP / HAL Layer                           |
| clock, GPIO, USART1/2, SPI2, external CSN, IRQ priorities        |
+------------------------------+-----------------------------------+
                               |
                               v
                         STM32C562RE
                               |
                               v
                            TMC6460

### Ownership rule

Only MotorServiceTask performs TMC6460 register transactions. This remains true during calibration. The calibration module is called synchronously from MotorServiceTask; it yields with FreeRTOS delays while moving/sampling, so DebugCommand, Diagnostics and Heartbeat continue running.

## 3. FreeRTOS task architecture

| Task | Priority | Responsibility |
| --- | --- | --- |
| MotorService | 4 | Sole TMC6460 owner, command execution, 400 Hz runtime position sampling, runs calibration state sequence |
| DebugCommand | 3 | USART2 command RX, pushes normal commands to MotorService queue, can set calibration abort flag |
| Heartbeat | 1 | PA5 heartbeat |
| Diagnostics | 1 | Periodic cached status output; never touches the TMC bus directly |

Normal runtime position polling remains 400 Hz. Calibration uses a dedicated 100 Hz motion sample rate because endpoint detection requires deterministic low-rate filtering rather than maximum bus throughput.

## 4. Debug command set

| Character | Action |
| --- | --- |
| A | Select/configure 10 kg actuator |
| B | Select/configure 20 kg actuator |
| C | Select/configure 35 kg actuator |
| V | CW run |
| W | CCW run |
| Z | Stop/hold; during calibration it requests abort |
| G | Automatic 0 degree / 90 degree calibration |
| R | Refresh diagnostic snapshot |
| H or ? | Help |

G is accepted only after an actuator profile has been initialized successfully.

## 5. Calibration objective

The actuator has mechanical end stops corresponding to:

CCW end = physical 0 degrees.

CW end = physical 90 degrees.

The controller's Hall-derived POSITION_ACTUAL is incremental. Therefore calibration determines:

Current-session raw 0 degree endpoint.

Current-session raw 90 degree endpoint.

Signed 0-to-90 stroke counts.

Absolute stroke counts.

Direction sign of position count growth.

Endpoint repeatability/spread.

The reusable compile-time quantity is primarily STROKE_COUNTS plus DIRECTION_SIGN. Raw absolute endpoint numbers are not guaranteed to be identical after a power cycle. At boot, a homing operation is still required to establish a new runtime zero reference before converting position to absolute degrees.

## 6. Calibration configuration

Calibration policy is intentionally stored in config/actuator_calibration_config.h, not hardware_config.h. hardware_config.h remains the single STM32C5 board/peripheral configuration file.

Main calibration macro:

#define ACTUATOR_CALIBRATION_ITERATION_COUNT 5U

Other key values:

#define ACTUATOR_CALIBRATION_SAMPLE_RATE_HZ        100U
#define ACTUATOR_CALIBRATION_STARTUP_BLANKING_MS   300U
#define ACTUATOR_CALIBRATION_STALL_CONFIRM_MS      250U
#define ACTUATOR_CALIBRATION_POSITION_DELTA_COUNTS 8L
#define ACTUATOR_CALIBRATION_MAX_LEG_TIME_MS       30000U
#define ACTUATOR_CALIBRATION_SETTLE_TIME_MS        150U
#define ACTUATOR_CALIBRATION_ENDPOINT_AVG_SAMPLES  16U

Low calibration velocity is separately configurable for each actuator type. Default is 250000 raw for 10/20/35 kg in this release.

## 7. Calibration state sequence

G received
   |
   +--> verify actuator initialized
   |
   +--> CAL START
   |
   +--> CCW low speed ------------------------------+
   |                                                |
   |       detect stable mechanical stop            |
   |                                                v
   +<-------------------------- HOME physical 0 deg
   |
   |   Initial homing is NOT counted in statistics
   |
   +--> for iteration = 1..N
          |
          +--> CW low speed -> detect stop -> HOLD -> capture 90 deg
          |
          +--> delay
          |
          +--> CCW low speed -> detect stop -> HOLD -> capture 0 deg
          |
          +--> delay
   |
   +--> analyze N zero samples + N ninety samples
   |
   +--> median endpoints
   +--> endpoint spread
   +--> stroke min/max/average
   +--> signed/absolute stroke
   +--> repeatability validation
   |
   +--> COMPLETE; actuator remains held at physical 0 deg

## 8. End-stop detection algorithm

The calibration deliberately approaches each mechanical stop at low velocity. A stop is accepted only after the startup blanking interval and after a stable condition persists for the full confirmation time.

For each 100 Hz sample:

position_delta = abs(position[n] - position[n-1])
velocity_low   = abs(actual_velocity) <= threshold
position_stable= position_delta <= configured count threshold

Default endpoint decision has two paths:

FAST confirmation:     position_stable AND velocity_low for 250 ms
ROBUST fallback:       position_stable continuously for 600 ms

The code also supports actuator-specific torque corroboration. If a non-zero ACTUATOR_CAL_xx_TORQUE_STALL_RAW is configured, the condition becomes:

position_stable AND (velocity_low OR torque_high)

Torque thresholds default to zero (disabled) until validated for each mechanism. This avoids false end detection caused by normal load torque.

The stable condition must remain true for ACTUATOR_CALIBRATION_STALL_CONFIRM_MS (250 ms by default), not merely one sample.

## 9. Endpoint capture

When an end is detected:

TMC6460_MotorStopHold() is called.

Gate driver remains enabled.

Velocity target becomes zero.

Hold torque/flux limit remains non-zero.

Firmware waits for mechanical settle time.

Firmware averages ACTUATOR_CALIBRATION_ENDPOINT_AVG_SAMPLES position reads.

This avoids using the first impacted/bouncing sample as the endpoint.

## 10. Statistical result

For each end, v11 stores all N samples and calculates:

Arithmetic average.

Median (used as the recommended endpoint).

Min-to-max spread.

For every iteration it also calculates stroke magnitude:

stroke[i] = abs(position90[i] - position0[i])

The final result includes:

zero_raw = median zero endpoint.

ninety_raw = median 90 endpoint.

stroke_signed_counts = ninety_raw - zero_raw.

stroke_counts = absolute signed stroke.

direction_sign = +1 or -1.

zero/90 spread.

stroke min/max/average.

Calibration fails as unstable if endpoint or stroke variation exceeds the configured percentage tolerance.

## 11. Generated compile-time values

After a successful calibration, the debug UART prints ready-to-paste macros, for example:

#define ACT_CAL_10KG_VALID 1U
#define ACT_CAL_10KG_ZERO_RAW_REFERENCE -12530L
#define ACT_CAL_10KG_NINETY_RAW_REFERENCE 2867412L
#define ACT_CAL_10KG_STROKE_COUNTS 2879942UL
#define ACT_CAL_10KG_DIRECTION_SIGN 1L

Paste them into config/actuator_calibration_config.h after validating the result on the bench.

Important: ZERO_RAW_REFERENCE and NINETY_RAW_REFERENCE are lab/session references. They must not be treated as absolute retained position after reset. Use stored stroke counts only after homing the actuator to 0 degrees in the current boot session.

## 12. Position-to-angle conversion

After G succeeds in the current session:

angle_deg_x100 =
    (position_raw - zero_raw) * 9000 / stroke_signed_counts

The diagnostic field angle= therefore reports values such as:

0.00
45.13
90.02

The function does not clamp the result so that mechanical overtravel or reference errors remain visible during validation.

## 13. Safety behavior

Calibration uses low velocity.

Every leg has a 30 second timeout.

Any TMC communication failure stops the calibration and requests HOLD.

Z during calibration sets an abort flag; the calibration loop checks it every sample and immediately transitions to HOLD.

Normal A/B/C/V/W/G/R commands are rejected while calibration is active so they cannot queue behind a long calibration operation.

The calibration path never deliberately writes torque/flux target to zero.

The gate driver remains enabled in hold unless the TMC itself reports a fault.

Because calibration intentionally finds physical hard stops, first validation should be performed with conservative supply/current limits and monitored hardware.

## 14. Calibration status in diagnostics

Additional fields are appended to the existing STAT line:

calAct=1       calibration currently running
calValid=0/1   valid result available in current session
calSt=N        calibration status enum
calIt=N        current/recent iteration
cal0=...       most recent/final zero endpoint
cal90=...      most recent/final ninety endpoint
stroke=...     final absolute stroke counts
angle=xx.xx    current-session calculated angle

Calibration status values:

| Value | Meaning |
| --- | --- |
| 0 | OK |
| 1 | Invalid parameter |
| 2 | Actuator not initialized |
| 3 | TMC communication/error |
| 4 | Leg timeout |
| 5 | Repeatability/consistency failure |
| 6 | User abort |

## 15. Source tree ownership

project/
  config/
    hardware_config.h                 STM32C5 hardware/RTOS only
    actuator_calibration_config.h     calibration policy/results

  bsp/
    bsp.c/h                           GPIO/RCC/UART/SPI hardware

  drivers/
    uart/uart_iface.c/h               generic interrupt UART
    spi/spi_iface.c/h                 generic interrupt SPI + external CS

  middleware/tmc6460/
    tmc6460_transport.c/h             UART/SPI TMC protocol transport
    tmc6460.c/h                       register API
    tmc6460_registers.h               register addresses + proven values
    tmc6460_motor.c/h                 configure/run/hold/motion sampling

  application/
    actuator_profile.c/h              actuator type parameters
    actuator_calibration.c/h          auto-calibration algorithm
    app.c/h                           RTOS-facing application service

  generated/middleware/
    mx_freertos_app.c/h               task creation only

## 16. Bench validation procedure

Build and flash v11.

Confirm normal v10.4 behavior first: A, V, Z, W, Z.

Return to a safe mid position if needed.

Send A (or B/C) to initialize selected actuator.

Send G once.

Observe CAL: messages and mechanical movement.

Confirm initial CCW homing reaches physical 0 degrees.

Confirm each iteration travels 0 -> 90 -> 0.

Confirm no unexpected high-speed impacts.

Confirm final output reports five samples per endpoint and prints macros.

Check endpoint spread and stroke spread are acceptably small.

Manually move after calibration and confirm angle= tracks approximately 0..90 degrees.

Copy validated result macros into actuator_calibration_config.h if desired.

## 17. Future extensions

This design intentionally leaves room for:

boot-time homing using compiled stroke values;

soft-stop/creep start derived as a percentage/count distance from each end;

flash/NVM persistence instead of manually copied macros;

actuator-specific torque-based hard-stop confirmation;

automatic calibration data CRC/versioning;

supervisory-controller command instead of debug UART G;

performance cycling between calibrated software limits.

# Appendix A - Calibration Macro Quick Reference

| Macro | Default | Purpose |
| --- | --- | --- |
| ACTUATOR_CALIBRATION_ITERATION_COUNT | 5 | Number of measured 0->90->0 cycles |
| ACTUATOR_CALIBRATION_SAMPLE_RATE_HZ | 100 | Sampling during calibration |
| ACTUATOR_CALIBRATION_STARTUP_BLANKING_MS | 300 | Ignore transient stall criteria after movement starts |
| ACTUATOR_CALIBRATION_STALL_CONFIRM_MS | 250 | Fast stable position + low velocity confirmation |
| ACTUATOR_CALIBRATION_POSITION_ONLY_CONFIRM_MS | 600 | Fallback if velocity feedback remains noisy |
| ACTUATOR_CALIBRATION_POSITION_DELTA_COUNTS | 8 | Maximum per-sample movement considered stationary |
| ACTUATOR_CALIBRATION_MAX_LEG_TIME_MS | 30000 | Per-leg safety timeout |
| ACTUATOR_CALIBRATION_ENDPOINT_AVG_SAMPLES | 16 | Endpoint averaging count |